libevent使用文档

版本：(wifi_video_master)https://gitee.com/Jieli-Tech/fw-AC792_SDK.git

板级：AC7926AF开发板

环境：window+windsurf(vscode)

vscode终端

  - cd tools

  - .\make_prompt.bat

  - make ac792n_wifi_camera

- 添加头文件和库

在wifi_camera/board/wl83/Makefile

- 依赖lwip库，lwip需要重新编译，需要loop和ip6的支持

- #define CONFIG_LWIP_IPV6_ENABLE

- #define LWIP_NETIF_LOOPBACK 1

- #define LWIP_HAVE_LOOPIF    1

- 测试例子,需要留意，必须要lwip初始化之后，调用libevent_timer_test

- /* Timer callback function */

- static void timer_test_cb(evutil_socket_t fd, short events, void *arg)

- {

- struct event *ev = arg;

- static int count = 0;

- printf("Timer triggered #%d, event type: %s%s%s\n",

- ++count,

- (events & EV_TIMEOUT) ? "TIMEOUT " : "",

- (events & EV_READ)    ? "READ " : "",

- (events & EV_WRITE)   ? "WRITE " : "");

- /* Stop event loop after 5 timer events */

- if (count >= 5) {

- event_base_loopbreak(event_get_base(ev));

- printf("Timer reached 5 triggers, exiting event loop\n");

- }

- }

- /* libevent timer test function */

- void libevent_timer_test(void)

- {

- struct event_base *base;

- struct event *ev = NULL;

- struct timeval tv;

- /* Initialize libevent */

- base = event_base_new();

- if (!base) {

- printf("Failed to initialize libevent\n");

- return;

- }

- printf("Using libevent backend method: %s\n", event_base_get_method(base));

- /* Create persistent timer */

- ev = event_new(base, -1, EV_PERSIST, timer_test_cb, event_self_cbarg());

- if (!ev) {

- printf("Failed to create timer event\n");

- event_base_free(base);

- return;

- }

- /* Set 1-second interval */

- evutil_timerclear(&tv);

- tv.tv_sec = 1;

- tv.tv_usec = 0;

- if (event_add(ev, &tv) < 0) {

- printf("Failed to add timer event\n");

- event_free(ev);

- event_base_free(base);

- return;

- }

- printf("Periodic timer added, will trigger every 1 second\n");

- printf("Starting event loop...\n");

- /* Start event loop until no events or explicitly broken */

- event_base_dispatch(base);

- /* Clean up resources */

- event_free(ev);

- event_base_free(base);

- printf("Event loop exited, timer test completed\n");

- }

- int getpid(void)

- {

- return 0;

- }

- 测试debug，这里循环了5次的例子